EASTERN INTERNATIONAL UNIVERSITY SCHOOL OF COMPUTING AND INFORMATION TECHNOLOGY

aa

Practice Assignment – Quarter …, 20…-20… Course Name: Special Topic 1

Course Code: CSE 484

Student’s Full Name:  Hà Quang Minh

Student ID:  1931220012

Lab 6

Background

This 3-hour lab applies model-driven automation on a Cisco CSR1000v using RESTCONF and NETCONF (ncclient), with a quick CLI parity check (Netmiko + TextFSM). You’ll enable the right services, fetch structured data, make safe transactional changes, and capture evidence for your report.

Required Resources:

- 1 PC (Linux/macOS/WSL recommended) with Python 3.9+ and VS Code

- A CSR1000v (or IOS-XE device) reachable from your PC

- Internet access to install packages: requests, ncclient, lxml, netmiko

- Credentials with privilege 15 on the CSR

# Problem:

Part 1 — Connectivity & Feature Enablement

# Enable services on CSR (CLI):

conf t

username cisco privilege 15 secret cisco ip domain-name lab.local

crypto key generate rsa modulus 2048 ip http secure-server

ip http authentication local restconf

netconf-yang line vty 0 4

transport input ssh

login local end

# Verify reachability: ping <CSR_IP> and ssh cisco@<CSR_IP> (port 22).

- Create venv + install libs

python -m venv .venv && source .venv/bin/activate pip install requests ncclient lxml netmiko

Evidence: screenshot of SSH login; command outputs confirming services.

Part 2 — RESTCONF Reads & Patch

Goal: pull JSON, then make a small, safe change.

# GET all interfaces (JSON):

curl -k -u <<admin>>:<<password_of_admin>> -H "Accept: application/yang-data+json" "https://<CSR_IP>/restconf/data/ietf- interfaces:interfaces/interface"

# GET one interface and operational state.

- Config:

.../restconf/data/ietf-interfaces:interfaces/interface=GigabitEthernet1

- State:

.../restconf/data/ietf-interfaces:interfaces-state/interface=GigabitEthernet1

# PATCH a description on Gi1.

curl -k -u cisco:cisco -X PATCH	-H "Content-Type: application/yang- data+json"

"https://<CSR_IP>/restconf/data/ietf- interfaces:interfaces/interface=GigabitEthernet1"	-d '{ "ietf- interfaces:interface": { "name":"GigabitEthernet1", "description":"LAB-Gi1", "type":"iana-if-type:ethernetCsmacd", "enabled": true } }'

- Verify: RESTCONF GET again and CLI “show run int g1 | i description”.

Evidence: JSON snippets before/after changes made; CLI verification.

Part 3 — NETCONF with ncclient

Goal: discover models; do a transactional hostname change.

# Discover capabilities.

# discover_caps.py

from ncclient import manager

with manager.connect(host="<CSR_IP>", port=830, username="cisco", password="cisco",

hostkey_verify=False, look_for_keys=False, allow_agent=False) as m:

for cap in m.server_capabilities: print(cap)

# Safe config flow (lock → load → validate → commit → unlock).

# set_hostname_nc.py

from ncclient import manager

from ncclient.xml_ import new_ele, sub_ele, to_xml

cfg = new_ele("config")

native = sub_ele(cfg, "native", xmlns="http://cisco.com/ns/yang/Cisco-IOS- XE-native")

sub_ele(native, "hostname").text = "CSR-DEMO"

with manager.connect(host="<CSR_IP>", port=830, username="cisco", password="cisco",

hostkey_verify=False, look_for_keys=False, allow_agent=False) as m:

target = "running" # use "candidate" if supported on your image m.lock(target)

try:

m.edit_config(target=target, config=to_xml(cfg))

if any(":validate" in c for c in m.server_capabilities):

m.validate()

if target == "candidate": m.commit()

finally:

m.unlock(target)

- Verify: RESTCONF .../native/hostname (or CLI show run | i hostname). Evidence: capability list, NETCONF reply XML, verification output.

Part 4 —Netmiko

Goal: CLI automation: structured parsing, config with verification, concurrency, logging, error- handling.

# Set env vars (safer than hard-coding):

Create an .env file with following contents:

export NET_HOST=<CSR_IP>

export NET_USERNAME=<user-name> export NET_PASSWORD=<password> #export NET_ENABLE=cisco (optional)

# Gather structured data (TextFSM) + anti-paging + prompts/modes

# nm_gather.py import os, json

from netmiko import ConnectHandler

dev = {

"device_type": "cisco_ios", "host": os.getenv("NET_HOST"),

"username": os.getenv("NET_USERNAME"), "password": os.getenv("NET_PASSWORD"), "secret": os.getenv("NET_ENABLE"),

"session_log": "session_csr.txt", # raw send/recv transcript

}

with ConnectHandler(**dev) as net:

print("Login prompt:", net.find_prompt()) # e.g., "CSR1kv>"

net.enable()	# -> privileged "#" net.send_command("terminal length 0")	# disable paging

brief = net.send_command("show ip interface brief", use_textfsm=True)

ver	= net.send_command("show version") print(json.dumps(brief, indent=2)[:500]) open("cli_show_ip_int_brief.json", "w").write(json.dumps(brief,

indent=2))

open("cli_show_version.txt", "w").write(ver)

Evidence: image of cli_show_ip_int_brief.json, cli_show_version.txt

# Handling slow devices & flaky links

# nm_slow_device.py

from netmiko import ConnectHandler import os

dev = {"device_type":"cisco_ios","host":os.getenv("NET_HOST"),

"username":os.getenv("NET_USERNAME"),"password":os.getenv("NET_PASSWO RD"),

"secret":os.getenv("NET_ENABLE")} with ConnectHandler(**dev) as net:

net.enable()

net.global_delay_factor = 2	# make all ops more patient out = net.send_command("show tech-support", delay_factor=4,

read_timeout=120) print(out[:800])

Evidence: image of the print command

# Config from file + per-device logs

Create a txt file named changes.txt containing some line as follows:

logging buffered 19999 no ip http server

Create and run a python file:

# nm_from_file.py

from netmiko import ConnectHandler import os, time

cmdfile = "changes.txt" # create with a few lines, e.g., "service timestamps debug datetime msec"

dev = {"device_type":"cisco_ios","host":os.getenv("NET_HOST"),

"username":os.getenv("NET_USERNAME"),"password":os.getenv("NET_PASSWO RD"),

"secret":os.getenv("NET_ENABLE"), "session_log":"session_apply.txt"}

with ConnectHandler(**dev) as net: net.enable()

out = net.send_config_from_file(cmdfile) open("cfg_from_file.txt","w").write(out)

Evidence: image of the cfg_from_file.txt file

My Solution:

ssh -oKexAlgorithms=+diffie-hellman-group14-sha1 cisco@192.168.74.152

Pass : cisco123!

Part 3:

PART 4:

PART 5:

Submission: Must include:

- a pdf report file containing your information (student id, name), and images of the diagrams, codes, answers, evident,…

- and all source code files (if any)

in a zipped (.zip or *.rar) file to Moodle