# Tab 1

Lab 6

Topology:

- DEVAS (Linux/WSL, Python 3.9+, VS Code) → CSR1000v (192.168.74.152) NAT

- SSH / HTTPS reachability cần ok

Part 1: Connectivity & Feature Enablement

Mục tiêu: Mở các service cần thiết, verify kết nối.

Commands trên CSR:

conf t username cisco privilege 15 secret cisco ip domain-name lab.local crypto key generate rsa modulus 2048 ip http secure-server ip http authentication local restconf netconf-yang line vty 0 4 transport input ssh login local end

Check:

ping 192.168.74.152

ssh cisco@cisco123!

Tạo virtualenv + cài thư viện:

python -m venv .venv source .venv/bin/activate pip install requests ncclient lxml netmiko

Evidence: SSH login, output confirm service enable.

Part 2 — RESTCONF (GET / PATCH)

Mục tiêu: Lấy dữ liệu JSON, sửa config an toàn.

GET tất cả interface

curl -k -u cisco:cisco123! -H "Accept: application/yang-data+json" \ "https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface"

GET một interface / trạng thái vận hành:

# Config .../interfaces/interface=GigabitEthernet1 # State .../interfaces-state/interface=GigabitEthernet1

PATCH description Gi1:

curl -k -u cisco:cisco -X PATCH \ -H "Content-Type: application/yang-data+json" \ "https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface=GigabitEthernet1" \ -d '{ "ietf-interfaces:interface": { "name":"GigabitEthernet1", "description":"LAB-Gi1", "type":"iana-if-type:ethernetCsmacd", "enabled": true } }'

Verify:

GET lại JSON

CLI: show run int g1 | i description

Evidence: JSON trước/sau, CLI output.

PART 3 : NETCONF với ncclient

Mục tiêu: Discover YANG models, hostname change an toàn.

Discover capabilities:( Khám phá khả năng )

File 1 — discover_caps.py

Chức năng: Liệt kê tất cả NETCONF capabilities của CSR.

Tạo file:

cd ~/lab_netconf nano discover_caps.py

Content:

from ncclient import manager with manager.connect( host="192.168.74.152", port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: print("NETCONF Capabilities:") for cap in m.server_capabilities: print(cap)

Chạy file & lưu evidence:

python discover_caps.py ¦ tee netconf_caps.txt

netconf_caps.txt là bằng chứng list capabilities.

File 2 — set_hostname_nc.py

Chức năng: Đổi hostname theo flow NETCONF: lock → edit → validate → commit → unlock

nano set_hostname_nc.py

from ncclient import manager from ncclient.xml_ import new_ele, sub_ele, to_xml cfg = new_ele("config") native = sub_ele(cfg, "native", xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native") sub_ele(native, "hostname").text = "CSR-DEMO" with manager.connect( host="192.168.74.152", port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: target = "running" m.lock(target) try: # Edit config m.edit_config(target=target, config=to_xml(cfg)) # Optional validate if device supports if any(":validate" in c for c in m.server_capabilities): m.validate() # Commit changes m.commit() print("Hostname changed successfully.") finally: m.unlock(target)

Chạy & verify:

python set_hostname_nc.py # CLI verify: ssh cisco@192.168.74.152 show run ¦ i hostname # RESTCONF verify: curl -k -u cisco:cisco -H "Accept: application/yang-data+json" \ "https://192.168.74.152/restconf/data/native/hostname"

Evidence: XML reply từ ncclient + CLI / RESTCONF output.

Part 4 — Netmiko (CLI Automation)

Mục tiêu: CLI automation, structured parsing (TextFSM), logging, config from file.

File 1 — .env

Chức năng: Lưu thông tin login an toàn.

nano ~/.env

export NET_HOST=192.168.74.152 export NET_USERNAME=cisco export NET_PASSWORD=cisco123! export NET_ENABLE=cisco

Load env:

source ~/.env

File 2 — nm_gather.py

Chức năng: Lấy dữ liệu cấu hình có cấu trúc, parse TextFSM, lưu file JSON + txt.

nano nm_gather.py

Content:

from netmiko import ConnectHandler import os, json dev = { "device_type": "cisco_ios", "host": os.getenv("NET_HOST"), "username": os.getenv("NET_USERNAME"), "password": os.getenv("NET_PASSWORD"), "secret": os.getenv("NET_ENABLE"), "session_log": "session_csr.txt" } with ConnectHandler(**dev) as net: net.enable() net.send_command("terminal length 0") # disable paging # Show IP interface brief structured brief = net.send_command("show ip interface brief", use_textfsm=True) ver = net.send_command("show version") # Lưu file with open("cli_show_ip_int_brief.json", "w") as f: json.dump(brief, f, indent=2) with open("cli_show_version.txt", "w") as f: f.write(ver) print("Data gathered, files saved.")

python nm_gather.py

Evidence: cli_show_ip_int_brief.json + cli_show_version.txt

File 3 — nm_slow_device.py

Chức năng: Handle devices chậm / link không ổn định.

nano nm_slow_device.py

from netmiko import ConnectHandler import os dev = { "device_type": "cisco_ios", "host": os.getenv("NET_HOST"), "username": os.getenv("NET_USERNAME"), "password": os.getenv("NET_PASSWORD"), "secret": os.getenv("NET_ENABLE") } with ConnectHandler(**dev) as net: net.enable() net.global_delay_factor = 2 out = net.send_command("show tech-support", delay_factor=4, read_timeout=120) with open("tech_support_partial.txt", "w") as f: f.write(out[:2000]) # chỉ lấy 2k ký tự đầu để dễ đọc print("Output saved.")

File 4 — nm_from_file.py

Chức năng: Apply config từ file txt + session log.

Tạo file changes.txt:

nano changes.txt

Nội dung ví dụ:

logging buffered 19999

no ip http server

service timestamps debug datetime msec

service timestamps log datetime msec

Tạo file Python:

nano nm_from_file.py

from netmiko import ConnectHandler import os cmdfile = "changes.txt" dev = { "device_type": "cisco_ios", "host": os.getenv("NET_HOST"), "username": os.getenv("NET_USERNAME"), "password": os.getenv("NET_PASSWORD"), "secret": os.getenv("NET_ENABLE"), "session_log": "session_apply.txt" } with ConnectHandler(**dev) as net: net.enable() out = net.send_config_from_file(cmdfile) with open("cfg_from_file.txt", "w") as f: f.write(out) print("Configuration applied, check cfg_from_file.txt and session_apply.txt")

python3 nm_from_file.py

Evidence: cfg_from_file.txt, session_apply.txt

Lab 7 NETWORK AUTOMATION WITH ANSIBLE (CSR1000v)

PHẦN 1 — CÀI ĐẶT & KIỂM TRA (INSTALL & VERIFY)

Goal: Làm cho Ansible nói chuyện được với Router CSR1000v.

python -m venv .venv source .venv/bin/activate pip install --upgrade pip pip install ansible ansible-pylibssh ansible-lint

Cài module mạng Cisco

ansible-galaxy collection install cisco.ios ansible.netcommon

Tạo cấu trúc thư mục

ansible-netlab/ ├── ansible.cfg ├── inventories/ │ ├── hosts │ └── host_vars/ │ └── csr1.yml └── playbooks/ └── sanity.yml

4️⃣ File ansible.cfg

[defaults] inventory = inventories/hosts host_key_checking = False stdout_callback = yaml gathering = explicit timeout = 30 retry_files_enabled = False [persistent_connection] connect_timeout = 30 command_timeout = 60

5️⃣ File inventories/hosts

Chỉnh file hosts với nano :

[ios] csr1 ansible_host=192.168.74.152

Ip của Router CSR1000v

6️⃣ File inventories/host_vars/csr1.yml

ansible_user: cisco ansible_password: cisco123! ansible_become: true ansible_become_method: enable ansible_become_password: cisco ansible_network_os: cisco.ios.ios ansible_connection: ansible.netcommon.network_cli

7️⃣ Playbook sanity check — playbooks/sanity.yml

- name: Sanity check IOS reachability hosts: ios gather_facts: no tasks: - name: Facts via ios_facts cisco.ios.ios_facts: - name: Show version cisco.ios.ios_command: commands: ["show version"] register: showver - name: Print model & version debug: msg: "Model={{ ansible_net_model }} Version={{ ansible_net_version }}"

📦 PHẦN 2 — BACKUP & IDEMPOTENT CONFIG

1️⃣ Backup config

File: playbooks/backup.yml

- name: Backup running-config hosts: ios gather_facts: no tasks: - name: Backup config (per-device file) cisco.ios.ios_config: backup: true backup_options: dir_path: backups

Chạy:    ansible-playbook playbooks/backup.yml

2️⃣ Idempotent config

File: playbooks/base_config.yml

- name: Base hardening (idempotent) hosts: ios gather_facts: no tasks: - name: Global settings cisco.ios.ios_config: lines: - service timestamps debug datetime msec - service timestamps log datetime msec - logging buffered 19999 - ip ssh version 2 - name: VTY lines cisco.ios.ios_config: parents: line vty 0 4 lines: - exec-timeout 10 0 - transport input ssh - login local

Chạy 2 lần — lần 2 phải ok (no changes) → đúng nghĩa "idempotent".

3️⃣ Backup có điều kiện (Self-task)

Backup khi hostname chứa “LAB”

File: playbooks/backup_lab_only.yml

- name: Backup only if hostname has LAB hosts: ios gather_facts: no tasks: - name: Gather facts cisco.ios.ios_facts: - name: Conditional backup cisco.ios.ios_config: backup: true backup_options: dir_path: backups when: "LAB" in ansible_net_hostname

🧠 PHẦN 3 — CONDITIONS & FACTS

File: playbooks/cond.yml

- name: Conditional hostname suffix hosts: ios gather_facts: no tasks: - name: Gather facts cisco.ios.ios_facts: - name: Append -LAB to hostname when needed cisco.ios.ios_config: lines: - "hostname {{ ansible_net_hostname }}-LAB" when: ansible_net_hostname is defined and not ansible_net_hostname.endswith('-LAB') - name: Show final hostname cisco.ios.ios_command: commands: ["show run ¦ include ^hostname"] register: h - debug: var=h.stdout_lines

📌 Ý nghĩa:

Điều kiện when → tránh chạy lại nhiều lần → tránh phá config.

🔁 PHẦN 4 — LOOPS

1️⃣ Loop tạo VLANs

File: playbooks/vlans.yml

- name: Create VLANs via loop hosts: ios gather_facts: no vars: vlans: - {id: 10, name: APP} - {id: 20, name: DB} - {id: 30, name: WEB} tasks: - name: Ensure VLANs exist cisco.ios.ios_config: lines: - "vlan {{ item.id }}" - " name {{ item.name }}" loop: "{{ vlans }}"

2️⃣ Dictionary loop cấu hình interface

File: playbooks/intfs.yml

- name: Access ports via dict loop hosts: ios gather_facts: no vars: access_ports: - { name: GigabitEthernet1, vlan: 10, desc: "APP Uplink" } - { name: GigabitEthernet2, vlan: 20, desc: "DB Uplink" } tasks: - name: Configure access interface cisco.ios.ios_config: parents: "interface {{ item.name }}" lines: - "description {{ item.desc }}" - "switchport mode access" - "switchport access vlan {{ item.vlan }}" - "no shutdown" loop: "{{ access_ports }}"

# Understand

TỔNG QUAN CHIẾN LƯỢC – HIỂU CHO GỌN:

Nguyên cái lab chia làm 4 con đường tự động hóa:

RESTCONF → nói chuyện với router bằng HTTP + JSON

NETCONF (ncclient) → nói chuyện bằng XML + RPC

Netmiko → nói chuyện bằng gõ CLI như con người

Ansible → nói chuyện bằng playbook YAML, tự động hóa cấp doanh nghiệp

PHẦN 5 — ANSIBLE (THE BIG BOSS)

Đây là automation cấp doanh nghiệp.

Mỗi phần của Ansible = 1 chức năng khác nhau:

sanity.yml → kiểm tra kết nối

backup.yml → backup config

base_config.yml → cấu hình idempotent

vlans.yml → loop tạo VLAN

intfs.yml → loop cấu hình interface