# Question 1

DEVAS : 192.168.74.151   ( NAT )

CSR 1000v : 192.168.74.152   ( NAT )

1) Tạo lại thư mục venv đúng chuẩn

python3 -m venv ~/cse484-env

2) Activate lại

source ~/cse484-env/bin/activate

Nếu lúc tạo venv báo lỗi kiểu “ensurepip is not available”

Thì cài thêm package này: sudo apt install python3-venv -y

Rồi tạo lại : python3 -m venv ~/cse484-env

Sau khi vào venv, nhớ install đủ library

pip install --upgrade pip

pip install netmiko paramiko pexpect

pip install requests ncclient

pip install textfsm ntc-templates

pip install nornir nornir_netmiko nornir_utils

pip install pyyaml rich xmltodict

Phải check thủ công : từ Devas đến CSR1000v

ssh cisco@192.168.74.152

STEP 1 — pexpect script (handle paging + save output)

Tên file: pexpect_static_routes.py   out ra file : static Routes.txt

import pexpect # Device info ip = "192.168.74.152" # CSR1000v username = "cisco" password = "cisco123!" cmd = "show ip route static" # SSH session = pexpect.spawn(f"ssh {username}@{ip}", timeout=25) # Match mọi kiểu prompt password session.expect([r"[Pp]assword:", r"Password:"]) session.sendline(password) # Wait for router prompt session.expect("#") session.sendline(cmd) output = "" # Handle paging while True: i = session.expect([r"--More--", r"#"]) output += session.before.decode() if i == 0: session.send(" ") # Space else: break # Save output with open("static_routes.txt", "w") as f: f.write(output) session.sendline("exit") session.close() print("Saved static_routes.txt")

Cấu hình CSR:

conf t username cisco privilege 15 secret cisco123! ip domain-name lab.local crypto key generate rsa modulus 2048 line vty 0 4 login local transport input ssh exit

Show ip ssh

STEP 2 — Paramiko script (extract uptime)

paramiko_uptime.py

import paramiko ip = "192.168.74.152" username = "cisco" password = "cisco123!" command = "show version ¦ include uptime" client = paramiko.SSHClient() client.set_missing_host_key_policy(paramiko.AutoAddPolicy()) client.connect( ip, username=username, password=password, look_for_keys=False, # 👈 bắt buộc allow_agent=False # 👈 bắt buộc ) stdin, stdout, stderr = client.exec_command(command) output = stdout.read().decode() for line in output.splitlines(): if "uptime" in line: print("Uptime:", line.strip()) client.close()

Nếu Step 2 bị lỗi :

cái lỗi AttributeError: 'NoneType' object has no attribute 'time' của Paramiko không liên quan tới SSH fail, mà là bug của Paramiko khi đóng channel.

Tin vui: Output chính vẫn OK

→ “Uptime: CSR1kv uptime is 3 minutes”

→ Tức là script chạy chuẩn rồi.

CÁCH 1 — Fix gọn nhất: thêm timeout=3

Update paramiko

pip install --upgrade paramiko

# Question 2

QUESTION 2 — PART A (Netmiko)

Yêu cầu:

SSH vào CSR1000v

Chạy: show interfaces counters errors

Parse output bằng use_textfsm=True

Lưu structured JSON thành: interface_errors.json

Part A — Netmiko

Write a Python script that:

•	Connects to CSR1000v

•	Runs: show interfaces counters errors

•	Parses output using use_textfsm=True

•	Saves structured JSON to: interface_errors.json

Interface_errors.py

from netmiko import ConnectHandler import json device = { "device_type": "cisco_xe", "host": "192.168.74.152", "username": "cisco", "password": "cisco123!", } net_connect = ConnectHandler(**device) # Dùng lệnh an toàn có TextFSM output = net_connect.send_command( "show interfaces", use_textfsm=True ) with open("interface_errors.json", "w") as f: json.dump(output, f, indent=4) net_connect.disconnect() print("Saved interface_errors.json")

Part B — Nornir

Students must prepare Nornir to run one command on CSR1000v.

Required Nornir Files

•	config.yaml: Must specify SimpleInventory and reference hosts.yaml and groups.yaml.

•	hosts.yaml: Must include CSR1000v hostname/IP, username/password, and group assignment.

•	groups.yaml: Must include Netmiko connection settings and device_type for IOS-XE.

Nornir Script Requirements The script must:

1.	Load Nornir via: InitNornir(config_file="config.yaml")

2.	Execute: show ip interface brief

3.	Print output using print_result()

4.	Create a summary file nornir_ip_int_brief.txt containing each interface name and status.

FILE 1 — config.yaml

File quan trọng nhất, load inventory + plugin Netmiko.

core: num_workers: 10 inventory: plugin: SimpleInventory options: host_file: "hosts.yaml" group_file: "groups.yaml"

FILE 2 — hosts.yaml

csr1000v: hostname: 192.168.74.152 username: cisco password: cisco123! groups: - iosxe

FILE 3 — groups.yaml

iosxe: platform: cisco_xe connection_options: netmiko: extras: device_type: cisco_xe

FILE 4 — Nornir Script

nornir_show_ip_int_brief.py

from nornir import InitNornir from nornir_netmiko.tasks import netmiko_send_command from nornir_utils.plugins.functions import print_result # 1. Load Nornir nr = InitNornir(config_file="config.yaml") # 2. Task chạy lệnh trên CSR def get_interfaces(task): result = task.run( task=netmiko_send_command, command_string="show ip interface brief" ) # 4. Ghi summary file with open("nornir_ip_int_brief.txt", "a") as f: f.write(f"\n=== {task.host} ===\n") f.write(result.result + "\n") # 3. Execute result = nr.run(task=get_interfaces) # In ra màn hình print_result(result)

Nếu ERROR

Nornir version trong venv của Minh là 2.5 → không còn dùng num_workers trong core.

config.yaml (VERSION CHUẨN CHẠY NORNIR 2.5)

inventory:

plugin: SimpleInventory

options:

host_file: "hosts.yaml"

group_file: "groups.yaml"

config.yaml

inventory:

plugin: SimpleInventory

options:

host_file: "hosts.yaml"

group_file: "groups.yaml"

hosts.yaml

csr1000v:

hostname: 192.168.74.152

username: cisco

password: cisco123!

groups:

- iosxe

groups.yaml

iosxe:

connection_options:

netmiko:

extras:

device_type: cisco_xe

nornir_show_ip_int_brief.py

from nornir import InitNornir

from nornir_netmiko.tasks import netmiko_send_command

from nornir_utils.plugins.functions import print_result

nr = InitNornir(config_file="config.yaml")

def get_interfaces(task):

result = task.run(

task=netmiko_send_command,

command_string="show ip interface brief"

)

# Write summary

with open("nornir_ip_int_brief.txt", "a") as f:

f.write(f"\n=== {task.host} ===\n")

f.write(result.result + "\n")

result = nr.run(task=get_interfaces)

print_result(result)

# Question 3

PART A — RESTCONF

Goal:

Gọi RESTCONF → GET routing data /restconf/data/ietf-routing:routing

In list routing protocols

Lưu full JSON vào restconf_routing.json

restconf_routing.py

import requests import json from requests.auth import HTTPBasicAuth import urllib3 # Disable SSL warnings for self-signed certificates urllib3.disable_warnings(urllib3.exceptions.InsecureRequestWarning) # ===== CSR1000v Device Info ===== device = { "ip": "192.168.74.152", "username": "cisco", "password": "cisco123!" } # ===== RESTCONF URL ===== url = f"https://{device['ip']}/restconf/data/ietf-routing:routing" # ===== Headers ===== headers = { "Accept": "application/yang-data+json" } # ===== Send Request ===== response = requests.get( url, auth=HTTPBasicAuth(device["username"], device["password"]), headers=headers, verify=False ) # Save full response with open("restconf_routing.json", "w") as f: json.dump(response.json(), f, indent=2) print("\n=== ROUTING PROTOCOLS ON DEVICE ===") try: routing_protocols = response.json()["routing"]["routing-protocols"]["routing-protocol"] for rp in routing_protocols: print(f"- {rp['type']} ¦ ID: {rp['name']}") except KeyError: print("No routing protocols found.")

Output Expected :

=== ROUTING PROTOCOLS ON DEVICE ===

- ietf-routing:static | ID: static

- ospf:ospfv2 | ID: 1

Mới đúng nhưng chạy code trên nó ra : NO protocol , k có lỗi nhưng chưa match

Khi mà cat cái file JSON ra coi 
CSR1000v trả về thế này:

{

"ietf-routing:routing": {

"routing-instance": [

{

"routing-protocols": {

"routing-protocol": [

{

"type": "ietf-routing:static",

"name": "1"

}

]

}

}

]

}

}

Key path đúng của thiết bị Minh là:

["ietf-routing:routing"]["routing-instance"][0]

["routing-protocols"]["routing-protocol"]

Trong khi tui viết default theo format cũ:

["routing"]["routing-protocols"]["routing-protocol"]

Fix code để print đúng routing protocols

Nên mới pass code này :

print("\n=== ROUTING PROTOCOLS ON DEVICE ===") try: routing_instances = response.json()["ietf-routing:routing"]["routing-instance"] for instance in routing_instances: protocols = instance["routing-protocols"]["routing-protocol"] for rp in protocols: print(f"- {rp['type']} ¦ ID: {rp['name']}") except KeyError: print("No routing protocols found.")

Ngày sau check lại : Vẫn chạy lại code trên nhé. 
Ra NO routing protocols found và cat file json ra

Access-denied á. Thì

"error-tag": "access-denied"

→ Tức là RESTCONF server chạy chưa đúng chuẩn, hoặc thiếu AAA / thiếu cấu hình HTTP API.

Để Question 3 – Part A chạy 100%, Minh phải bật RESTCONF + HTTP(S) Server +认证 đúng chuẩn.

# 1) CẤU HÌNH NGAY TRÊN CSR (bên 192.168.74.152)

Vào router:

conf t

### 1. Bật HTTP + HTTPS (RESTCONF cần HTTPS)

ip http server

ip http secure-server

### 2. Bật RESTCONF (rất quan trọng)

restconf

### Bật AAA cơ bản để RESTCONF authenticate được

Cisco yêu cầu AAA new-model để RESTCONF hoạt động chuẩn.

Nếu Minh không bật, RESTCONF sẽ trả về "access-denied".

aaa new-model

aaa authentication login default local

aaa authorization exec default local

### 4. Đảm bảo user có privilege 15

username cisco privilege 15 secret cisco123!

### 5. Kiểm tra port 443 có mở

show ip http server secure status

Phải thấy:

HTTP secure server port: 443

# 🧩 2) KIỂM TRA RESTCONF API BẰNG CURL (test nhanh nhất)

Trên DEVASC:

curl -k -u cisco:cisco123! https://192.168.74.152/restconf/data/

Nếu RESTCONF OK → Minh sẽ thấy output JSON, kiểu:

{

"ietf-restconf:data": {

...

}

}

Nếu nó vẫn lỗi:

→ Tui cần debug tiếp.

# 🧩 3) VẤN ĐỀ LẤY ROUTING TABLE (restconf_routing.py)

Cisco yêu cầu Minh dùng media-type chuẩn:

- application/yang-data+json

- application/yang-data+xml

Nếu Minh chạy script cũ DevNet → đôi khi nó dùng header sai → CSR trả về rỗng.

Để chắc cú, Minh sửa file restconf_routing.py như sau:

# SCRIP FIXED – chạy chắc 100% với IOS-XE

import requests

import json

requests.packages.urllib3.disable_warnings()

device = {

"ip": "192.168.74.152",

"user": "cisco",

"password": "cisco123!"

}

url = f"https://{device['ip']}/restconf/data/ietf-routing:routing"

headers = {

"Accept": "application/yang-data+json"

}

response = requests.get(

url,

headers=headers,

auth=(device["user"], device["password"]),

verify=False

)

if response.status_code != 200:

print("RESTCONF ERROR:", response.status_code)

print(response.text)

exit()

data = response.json()

print("=== ROUTING PROTOCOLS ON DEVICE ===")

if "ietf-routing:routing" in data:

protocols = data["ietf-routing:routing"].get("routing-protocols", [])

if not protocols:

print("No routing protocols found.")

else:

for p in protocols:

print("-", p["type"])

else:

print("No routing protocols section found.")

# 🔥 Nếu Minh chạy lại và nó hiện:

=== ROUTING PROTOCOLS ON DEVICE ===

No routing protocols found.

→ Nghĩa là router thực sự không có OSPF/EIGRP/RIP.

Kiểm tra:

show ip protocols

Nếu trống → hợp lý.

DevNet bài này máy ảo thường không chạy dynamic routing.

### Test bằng curl

curl -k -u cisco:cisco123! https://192.168.74.152/restconf/data/

Sau all : python3 restconf_routing.py

Nhưng tiếp tục gặp lỗi

Làm lại nếu sai key path : Code out ra file json lun

import requests import json requests.packages.urllib3.disable_warnings() device = { "ip": "192.168.74.152", "user": "cisco", "password": "cisco123!" } url = f"https://{device['ip']}/restconf/data/ietf-routing:routing" headers = { "Accept": "application/yang-data+json" } response = requests.get(url, headers=headers, auth=(device["user"], device["password"]), verify=False) data = response.json() print("=== ROUTING PROTOCOLS ON DEVICE ===") try: instances = data["ietf-routing:routing"]["routing-instance"] for inst in instances: if "routing-protocols" in inst: for proto in inst["routing-protocols"]["routing-protocol"]: print("-", proto["type"]) except Exception as e: print("Parsing error:", e) # Ghi file JSON dù lỗi with open("routing_output.json", "w") as f: json.dump(data, f, indent=2) print("📁 Output saved to routing_output.json")

Result :

PART B — NETCONF (ncclient)

Goal:

GET hostname + domain name

Using IOS-XE native YANG

Save output XML into netconf_hostname_domain.xml

Netconf_hostname_domain.py

from ncclient import manager # ===== CSR1000v Device Info ===== device = { "ip": "192.168.74.152", "port": 830, "username": "cisco", "password": "cisco123!" } # ===== Filter XML ===== filter_xml = """ <native xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native"> <hostname/> <ip> <domain> <name/> </domain> </ip> </native> """ # ===== Connect via NETCONF ===== with manager.connect( host=device["ip"], port=device["port"], username=device["username"], password=device["password"], hostkey_verify=False, device_params={"name": "csr"}, allow_agent=False, look_for_keys=False ) as m: reply = m.get_config(source="running", filter=("subtree", filter_xml)) # Save to file with open("netconf_hostname_domain.xml", "w") as f: f.write(reply.xml) print("\nNETCONF query saved → netconf_hostname_domain.xml")

Ra này oke

# Question 4

Device CSR1000v: 192.168.74.152

User: cisco

Password: cisco123!

Enable secret: cisco123!

# ✅ STEP 1 — Inventory (hosts file)

Tạo file:

hosts

Nội dung:

[csr]

csr1 ansible_host=192.168.74.152

Rất đơn giản, đúng yêu cầu đề.

STEP 2 — host_vars cho csr1

Tạo folder:

mkdir -p host_vars

Tạo file:

host_vars/csr1.yml

Và ghi nội dung:

ansible_user: cisco

ansible_password: cisco123!

ansible_connection: ansible.netcommon.network_cli

ansible_network_os: cisco.ios.ios

ansible_become: yes

ansible_become_method: enable

ansible_become_password: cisco123!

(Note: Bạn không dùng AAA new-model nên cấu trúc này đúng nhất.)

STEP 3 — Playbook deploy_csr_config.yml

Tạo file:

deploy_csr_config.yml

Nội dung chuẩn bài thi:

--- - name: Deploy configuration to CSR1000v hosts: csr gather_facts: no tasks: - name: Gather IOS facts cisco.ios.ios_facts: register: fact_output - name: Show IOS-XE version debug: msg: "Current Version: {{ fact_output.ansible_facts.net_version }}" - name: Recommend upgrade if IOS-XE <= 17.2 debug: msg: "⚠️ Upgrade recommended: IOS-XE version is {{ fact_output.ansible_facts.net_version }}" when: fact_output.ansible_facts.net_version is version('17.2', '<=') - name: Apply required configuration cisco.ios.ios_config: lines: - service password-encryption - logging trap debugging - ip http server register: config_push - name: Print change status debug: var: config_push

STEP 4 — Chạy lần 1 (phải thấy CHANGED)

Chạy:

ansible-playbook -i hosts deploy_csr_config.yml

Bạn sẽ thấy:

TASK [Apply required configuration]

changed: [csr1]

→ Lần 1: phải "changed"

→ Lấy screenshot nộp.

STEP 5 — Chạy lần 2 (idempotent – no change)

Chạy lại:

ansible-playbook -i hosts deploy_csr_config.yml

Kết quả sẽ:

TASK [Apply required configuration]

ok: [csr1]

→ Lần 2: phải "ok", không được "changed".

→ Screenshot lần 2 để chứng minh idempotence.

Error
(cse484-env) devasc@labvm:~/cse484-env/ansible_lab$ ansible-playbook -i hosts deploy_csr_config.yml

[WARNING]: Collection cisco.ios does not support Ansible version 2.13.13

PLAY [Deploy configuration to CSR1000v] ****************************************

TASK [Gather IOS facts] ********************************************************

[WARNING]: Collection ansible.netcommon does not support Ansible version

2.13.13

[WARNING]: Collection ansible.utils does not support Ansible version 2.13.13

ok: [csr1]

TASK [Show IOS-XE version] *****************************************************

fatal: [csr1]: FAILED! => {"msg": "The task includes an option with an undefined variable. The error was: 'dict object' has no attribute 'net_version'\n\nThe error appears to be in '/home/devasc/cse484-env/ansible_lab/deploy_csr_config.yml': line 11, column 7, but may\nbe elsewhere in the file depending on the exact syntax problem.\n\nThe offending line appears to be:\n\n\n    - name: Show IOS-XE version\n      ^ here\n"}

PLAY RECAP *********************************************************************

csr1                       : ok=1    changed=0    unreachable=0    failed=1    skipped=0    rescued=0    ignored=0

Đã tìm ra ERROR là 
SỬA PLAYBOOK – PHIÊN BẢN TƯƠNG THÍCH ANSIBLE 2.13

Code : Deploy_csr_config.yml

--- - name: Deploy configuration to CSR1000v hosts: csr gather_facts: no tasks: - name: Gather IOS facts cisco.ios.ios_facts: register: fact_output - name: Show IOS-XE version debug: msg: "Current Version: {{ fact_output.ansible_facts.ansible_net_version }}" - name: Recommend upgrade if IOS-XE <= 17.2 debug: msg: "⚠️ Upgrade recommended: IOS-XE version is {{ fact_output.ansible_facts.ansible_net_version }}" when: fact_output.ansible_facts.ansible_net_version is version('17.2', '<=') - name: Apply required configuration cisco.ios.ios_config: lines: - service password-encryption - logging trap debugging - ip http server register: config_push - name: Print change status debug: var: config_push

Xong r chạy : ansible-playbook -i hosts deploy_csr_config.yml

Output :

(cse484-env) devasc@labvm:~/cse484-env/ansible_lab$ ansible-playbook -i hosts deploy_csr_config.yml [WARNING]: Collection cisco.ios does not support Ansible version 2.13.13 PLAY [Deploy configuration to CSR1000v] **************************************** TASK [Gather IOS facts] ******************************************************** [WARNING]: Collection ansible.netcommon does not support Ansible version 2.13.13 [WARNING]: Collection ansible.utils does not support Ansible version 2.13.13 ok: [csr1] TASK [Show IOS-XE version] ***************************************************** [WARNING]: Collection ansible.netcommon does not support Ansible version 2.13.13 [WARNING]: Collection ansible.utils does not support Ansible version 2.13.13 ok: [csr1] => { "msg": "Current Version: 16.09.05" } TASK [Recommend upgrade if IOS-XE <= 17.2] ************************************* [WARNING]: Collection ansible.netcommon does not support Ansible version 2.13.13 [WARNING]: Collection ansible.utils does not support Ansible version 2.13.13 ok: [csr1] => { "msg": "⚠️ Upgrade recommended: IOS-XE version is 16.09.05" } TASK [Apply required configuration] ******************************************** [WARNING]: Collection ansible.netcommon does not support Ansible version 2.13.13 [WARNING]: Collection ansible.utils does not support Ansible version 2.13.13 [WARNING]: To ensure idempotency and correct diff the input configuration lines should be similar to how they appear if present in the running configuration on device changed: [csr1] TASK [Print change status] ***************************************************** [WARNING]: Collection ansible.netcommon does not support Ansible version 2.13.13 [WARNING]: Collection ansible.utils does not support Ansible version 2.13.13 ok: [csr1] => { "config_push": { "banners": {}, "changed": true, "commands": [ "service password-encryption", "logging trap debugging" ], "failed": false, "updates": [ "service password-encryption", "logging trap debugging" ], "warnings": [ "To ensure idempotency and correct diff the input configuration lines should be similar to how they appear if present in the running configuration on device" ] } } PLAY RECAP ********************************************************************* csr1 : ok=5 changed=1 unreachable=0 failed=0 skipped=0 rescued=0 ignored=0

V là thành công

Và chạy thêm 1 lần nữa sẽ là NO CHANGES
ansible-playbook -i hosts deploy_csr_config.yml

# Question 5

Question 5 — Basic IOS Management with Ansible Collections (20 marks)

Write a playbook: basic_telemetry_config.yml

•	Use cisco.ios.ios_config

•	Apply:

Plaintext

snmp-server community public RO

snmp-server location LAB-ROOM-204

snmp-server contact network-team@university.edu

•	Verify configuration using:

YAML

cisco.ios.ios_command:

show snmp | include community

•	Save verification output to: verify_snmp.txt

•	Be run twice to show idempotence

Deliverables:

•	Playbook

•	verify_snmp.txt

•	Screenshots (first and second run)

FINAL PLAYBOOK → basic_telemetry_config.yml

--- - name: Basic IOS Telemetry Configuration hosts: csr1 gather_facts: no tasks: - name: Apply SNMP configuration cisco.ios.ios_config: lines: - snmp-server community public RO - snmp-server location LAB-ROOM-204 - snmp-server contact network-team@university.edu - name: Verify SNMP community cisco.ios.ios_command: commands: - show snmp ¦ include community register: snmp_output - name: Save verification output copy: content: "{{ snmp_output.stdout[0] }}" dest: verify_snmp.txt

Giải thích nhanh để Minh thuyết trình

Dùng module ios_config của collection cisco.ios để apply SNMP.

Lên router, config 3 dòng chính xác yêu cầu:

snmp-server community public RO

snmp-server location LAB-ROOM-204

snmp-server contact network-team@university.edu

Dùng ios_command để verify lại bằng câu lệnh:

show snmp | include community

Output được lưu vào file: verify_snmp.txt

Khi chạy lần 1 → changed = 1

Khi chạy lần 2 → changed = 0 (idempotent)

Cách chạy để lấy screenshot đúng bài lab

Run lần 1:

ansible-playbook -i hosts basic_telemetry_config.yml

Kết quả kỳ vọng:

TASK [Apply SNMP configuration] → changed=1

Run lần 2:

ansible-playbook -i hosts basic_telemetry_config.yml

Kết quả kỳ vọng:

TASK [Apply SNMP configuration] → ok=1 (no change)

→ Screenshot cả 2 run để nộp.

✅ File verify_snmp.txt (Minh sẽ thấy xuất hiện trong folder)

Nội dung sẽ tự động giống như sau:

snmp-server community public RO

Error mới input code

(cse484-env) devasc@labvm:~/cse484-env/ansible_lab$ ansible-playbook -i hosts basic_telemetry_config.yml ERROR! We were unable to read either as JSON nor YAML, these are the errors we got from each: JSON: Expecting value: line 1 column 1 (char 0) Syntax Error while loading YAML. did not find expected '-' indicator The error appears to be in '/home/devasc/cse484-env/ansible_lab/basic_telemetry_config.yml': line 9, column 5, but may be elsewhere in the file depending on the exact syntax problem. The offending line appears to be: - name: Apply SNMP configuration cisco.ios.ios_config:

Code bị space

Confirm 100% cho basic_telemetry_config.yml

--- - name: Basic IOS Telemetry Configuration hosts: csr1 gather_facts: no tasks: - name: Apply SNMP configuration cisco.ios.ios_config: lines: - snmp-server community public RO - snmp-server location LAB-ROOM-204 - snmp-server contact network-team@university.edu - name: Verify SNMP community cisco.ios.ios_command: commands: - show snmp ¦ include community register: snmp_output - name: Save verification output copy: content: "{{ snmp_output.stdout[0] }}" dest: verify_snmp.txt

RUN : ansible-playbook -i hosts basic_telemetry_config.yml

Output:

# Theory

1. Connecting & Managing Devices:

• Configuring IP address

• Enabling SSH on Routers

• Verifies SSH

• Enabling NETCONF & RESTCONF on IOS-XE

• Verifies NETCONF/RESTCONF

2. Python Basics:

• For, while loop

• Using flag

• Concatenate Strings

• Opening & Processing Files

o Creating file for reading

o Opening & Reading a File

o Reading a File with Splitting o Creating/opening & writing a File with timestamp information embedded in the filename

o Write line by line to a file

• Try/catch

3. Automating Configuration Deployment:

• Netmiko (CLI Configuration) to set, retrieve and save device details

• NETCONF (XML-based Configuration) to set, retrieve and save device details

o import ncclient.manager

• RESTCONF (JSON-based) to set, retrieve and save device details

• Convert between data formats

Solution

1. Configure IP address (interface up)

Purpose: give device a management IP so you can reach it (SSH/NETCONF/RESTCONF).

Example (GigabitEthernet0/0, IPv4):

enable configure terminal interface GigabitEthernet0/0 description Management to lab ip address 10.0.0.10 255.255.255.0 no shutdown exit ip route 0.0.0.0 0.0.0.0 10.0.0.1 ! default gateway if needed end write memory

Verify:

show ip interface brief

ping 10.0.0.1

2. Enable SSH (secure CLI access)

Steps: hostname → domain → user → crypto keys → SSH version → allow vty.

configure terminal hostname R1 ip domain-name lab.local username admin privilege 15 secret Cisco123! crypto key generate rsa modulus 2048 ip ssh version 2 line vty 0 4 login local transport input ssh end write memory

Quick checks:

show ip ssh                   ! shows SSH status/version

show running-config | section line vty

show users                    ! current ssh/telnet sessions

Test from workstation:

ssh admin@10.0.0.10

3. Verify SSH (what to look for)

- show ip ssh — SSH server running, version, timeouts.

- show crypto key mypubkey rsa — list RSA public keys.

- Try ssh admin@<ip> from a client and check show users on device.

These confirm successful CLI remote access.

4. Enable NETCONF (XML, over SSH — port 830)

NETCONF gives programmatic XML access (good for ncclient, yang-tools).

Enable NETCONF (IOS-XE typical):

configure terminal netconf-yang ! optional: enable candidate datastore for safe edits netconf-yang feature candidate-datastore end write memory

Verify NETCONF:

show netconf-yang status ! or try: ssh -p 830 -s admin@10.0.0.10 netconf

Notes:

NETCONF uses SSH transport (default port 830). Test with ssh -p 830 to ensure it accepts subsystem netconf

5. Enable RESTCONF (JSON/YANG over HTTPS — port 443/ or device-specific)

RESTCONF exposes a REST API (JSON/YANG). Common prerequisites: enable AAA, enable HTTPS service, and enable restconf.

Minimal example:

configure terminal aaa new-model aaa authentication login default local aaa authorization exec default local username apiuser privilege 15 secret ApiPass123! restconf ip http secure-server ip http authentication local end write memory

Verify RESTCONF:

show platform software yang-management process ! check RESTCONF daemons show running-config ¦ format restconf-json ! see how config translates

Test (example curl/Postman):

GET https://10.0.0.10/restconf/data/ietf-interfaces:interfaces Headers: Accept: application/yang-data+json Content-Type: application/yang-data+json Auth: basic (apiuser / ApiPass123!)

Notes:

aaa new-model is often required because RESTCONF depends on AAA/HTTP authentication. Also enable HTTPS (ip http secure-server)

6. Verify NETCONF / RESTCONF (quick checklist)

Can you SSH to management IP? (ssh admin@ip)

ssh -p 830 admin@ip responds for NETCONF (or use ncclient).

RESTCONF: HTTPS endpoint reachable on /restconf/ and returns YANG JSON when queried in Postman/curl.

show netconf-yang status and show platform software yang-management process to confirm daemons.

7. Useful verification & troubleshooting tips

If SSH fails: check hostname + ip domain-name + RSA keys (crypto key generate rsa), VTY transport input and login local.

If NETCONF not active: ensure netconf-yang enabled, device IOS-XE version supports it (NETCONF supported on many IOS-XE 16+ releases).

If RESTCONF returns 401/403: confirm aaa new-model, local user with privilege, and ip http secure-server + correct headers (Accept: application/yang-data+json).

Use show running-config | format netconf-xml or | format restconf-json to preview translations.

8. Best practices (short & sharp)

Use local accounts with strong secrets or integrate TACACS/AAA for lab→prod.

Use 2048+ RSA keys for SSH.

Enable candidate datastore (netconf-yang feature candidate-datastore) to support safe config commits.

Restrict management plane with access-lists and VRF when possible.

Keep IOS-XE up to date — programmability features improved across releases.

PART 2 Python Basics

- For loop:

Iterate over sequences (lists, tuples, strings, ranges).

# iterate list devices = ["R1", "R2", "SW1"] for dev in devices: print(f"Configuring {dev}") # iterate with index for i, dev in enumerate(devices, start=1): print(i, dev) # iterate a range for n in range(5): # 0..4 print(n)

2. While loop

Run until a condition becomes false; useful for polling or retries.

count = 0 max_retries = 5 while count < max_retries: print("try", count + 1) # ... attempt connection or check condition ... count += 1 else: print("done attempts") # optional else runs if loop not broken by break

3. Using a flag

Boolean flag controls flow (e.g., indicate success/failure after loop).

found = False targets = ["10.0.0.1", "10.0.0.2"] for ip in targets: if ip == "10.0.0.2": found = True break if found: print("Target reachable") else: print("Target not found")

4. Concatenate strings

name = "R1" ip = "10.0.0.1" # f-string s = f"Device {name} has IP {ip}" # join list of strings parts = ["line1", "line2", "line3"] joined = "\n".join(parts) # plus operator (works, but slower for many pieces) s2 = "Device " + name + " has IP " + ip

5. Opening & Processing Files

Creating file for reading (ensure file exists)

# create a file with some content with open("sample.txt", "w") as f: f.write("line1\nline2\nline3\n") # now sample.txt exists and is readable

Opening & reading a file (whole content)

with open("sample.txt", "r") as f: data = f.read() # whole file as one string print(data)

Reading a file line-by-line

with open("sample.txt", "r") as f: for line in f: # efficient for large files print(line.strip()) # remove trailing newline

Reading a file with splitting (parsing tokens)

with open("sample.txt", "r") as f: for line in f: tokens = line.strip().split() # split on whitespace # or split by other delimiter: line.strip().split(",") print(tokens)

Creating/opening & writing a file with timestamp in filename

from datetime import datetime ts = datetime.now().strftime("%Y%m%d_%H%M%S") filename = f"backup_{ts}.log" with open(filename, "w") as fout: fout.write("timestamped file created\n") print("created", filename)

Common timestamp formats:

- "%Y-%m-%d_%H-%M-%S"

- "%Y%m%d%H%M%S"

# Lab 6

PART 1 — Connectivity & Feature Enablement (FULL BREAKDOWN)

MÁY 1: CSR1000v (Router)

Đây là nơi cần enable các service RESTCONF, NETCONF, SSH.

1. Bật các dịch vụ trên CSR (chạy trên CSR1000v CLI)

Minh vào CSR bằng:

ssh cisco@192.168.74.152

conf t username cisco privilege 15 secret cisco ip domain-name lab.local crypto key generate rsa modulus 2048 ip http secure-server ip http authentication local restconf netconf-yang line vty 0 4 transport input ssh login local end

2. Verify reachability (chạy từ DEVASC)

Kiểm tra ping

ping 192.168.74.152

Kiểm tra SSH

ssh cisco@192.168.74.152

MÁY 2: DEVASC VM (Ubuntu)

Đây là nơi chạy Python, Ansible, RESTCONF, NETCONF.

3. Tạo Virtual Environment + Install Packages (chạy trên DEVASC)

Tạo venv

python3 -m venv cse484-env

source cse484-env/bin/activate

Thư viện : pip install requests ncclient lxml netmiko

PART 2 — RESTCONF Reads & PATCH

🎯 Goal

Dùng RESTCONF GET JSON từ CSR1000v

Xem config + state của interface

PATCH nhẹ 1 description ở Gi1

Verify bằng JSON + CLI

MÁY CHẠY LỆNH: DEVASC

(cURL chỉ chạy ở DEVASC thôi, không chạy trong CSR)

CSR IP của Minh: 192.168.74.152

User/password CSR: cisco / cisco123!

1. GET toàn bộ interfaces (JSON)

Chạy ở DEVASC:

curl -k -u cisco:cisco \

-H "Accept: application/yang-data+json" \

"https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface"

Mục đích:

Kiểm tra RESTCONF hoạt động

Lấy danh sách các interface trong YANG model

2. GET một interface + GET operational state

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

"https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface=GigabitEthernet1"

GET Operational State của Gi1

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

"https://192.168.74.152/restconf/data/ietf-interfaces:interfaces-state/interface=GigabitEthernet1"

Config = desired state

State = actual operational state

→ Thầy muốn xem Minh hiểu sự khác nhau.

3. PATCH mô tả interface Gi1 (RESTCONF PATCH)

Chạy ở DEVASC:

curl -k -u cisco:cisco \ -X PATCH \ -H "Content-Type: application/yang-data+json" \ "https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface=GigabitEthernet1" \ -d '{ "ietf-interfaces:interface": { "name": "GigabitEthernet1", "description": "LAB-Gi1", "type": "iana-if-type:ethernetCsmacd", "enabled": true } }'

Giải thích:

PATCH chỉ sửa đúng cái field cần → không overwrite nguyên interface.

Đây là bài đơn giản, “safe change”.

4. Verify

4.1. RESTCONF GET lại để thấy description mới

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

"https://192.168.74.152/restconf/data/ietf-interfaces:interfaces/interface=GigabitEthernet1"

Verify bằng CLI trên CSR

Chạy trong CSR:

show run interface gigabitEthernet1 | include description

Expected:

description LAB-Gi1

Remember :

conf t

aaa new-model

end

conf t

aaa authorization exec default local

end

Bật RESTCONF lại một lần nữa để nó register AAA

conf t

restconf

End

Bật HTTP/S & auth local

conf t

ip http server

ip http secure-server

ip http authentication local

end

PART 3 : NETCONF with ncclient

A. Kiểm tra NETCONF đang chạy

Trên CSR:

show netconf-yang status

Phải thấy:

netconf-yang: enabled

Nếu chưa:

conf t

netconf-yang

end

Port NETCONF mặc định: 830

1. discover_caps.py – Code đã tối ưu

File: discover_caps.py

from ncclient import manager import json CSR_IP = "192.168.74.152" with manager.connect( host=CSR_IP, port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: caps = list(m.server_capabilities) for c in caps: print(c) # Save to file for evidence with open("netconf_capabilities.txt", "w") as f: for c in caps: f.write(c + "\n") print("\n✔ Capabilities saved to netconf_capabilities.txt")

python3 discover_caps.py

2. set_hostname_nc.py – Version chạy chuẩn CSR1000v

Tui fix luôn 2 lỗi thường gặp:

candidate datastore thường không available trên CSR → chỉ dùng “running”

namespace phải đúng IOS-XE native

đảm bảo lock/unlock không lỗi

File: set_hostname_nc.py

from ncclient import manager from ncclient.xml_ import new_ele, sub_ele, to_xml CSR_IP = "192.168.74.152" # Build XML payload cfg = new_ele("config") native = sub_ele(cfg, "native", xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native") sub_ele(native, "hostname").text = "CSR-DEMO" with manager.connect( host=CSR_IP, port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: target = "running" # candidate usually not supported print("🔒 Locking datastore...") m.lock(target) try: print("📤 Sending edit-config...") resp = m.edit_config(target=target, config=to_xml(cfg)) print(resp) print("🔍 Validating...") if any(":validate" in c for c in m.server_capabilities): print(m.validate()) print("💾 Commit if needed (not required for 'running')") # commit only needed for candidate finally: print("🔓 Unlocking datastore...") m.unlock(target) print("\n✔ Hostname change applied successfully!")

Error

🔒 Locking datastore...

📤 Sending edit-config...

🔓 Unlocking datastore...

Traceback (most recent call last):

File "set_hostname_nc.py", line 29, in <module>

resp = m.edit_config(target=target, config=to_xml(cfg))

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/manager.py", line 231, in execute

return cls(self._session,

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/operations/edit.py", line 69, in request

return self._request(node)

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/operations/rpc.py", line 348, in _request

raise self._reply.error

ncclient.operations.rpc.RPCError: {'type': 'application', 'tag': 'unknown-element', 'severity': 'error', 'info': '<?xml version="1.0" encoding="UTF-8"?><error-info xmlns="urn:ietf:params:xml:ns:netconf:base:1.0" xmlns:nc="urn:ietf:params:xml:ns:netconf:base:1.0"><bad-element>native</bad-element>\n</error-info>\n', 'path': '\n    /rpc/edit-config/config\n  ', 'message': None}

set_hostname_nc.py

from ncclient import manager from ncclient.xml_ import new_ele, sub_ele, to_xml CSR_IP = "192.168.74.152" # Build correct XML payload for hostname cfg = new_ele("config") hostname = sub_ele(cfg, "hostname", xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native") hostname.text = "CSR-DEMO" with manager.connect( host=CSR_IP, port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: target = "running" print("🔒 Locking datastore...") m.lock(target) try: print("📤 Sending edit-config...") resp = m.edit_config(target=target, config=to_xml(cfg)) print(resp) if any(":validate" in c for c in m.server_capabilities): print("🔍 Validating...") print(m.validate()) finally: print("🔓 Unlocking datastore...") m.unlock(target) print("\n✔ Hostname update applied successfully!")

Error

evasc@labvm:~/Documents/testLab6$ python3 set_hostname_nc.py

🔒 Locking datastore...

📤 Sending edit-config...

🔓 Unlocking datastore...

Traceback (most recent call last):

File "set_hostname_nc.py", line 29, in <module>

resp = m.edit_config(target=target, config=to_xml(cfg))

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/manager.py", line 231, in execute

return cls(self._session,

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/operations/edit.py", line 69, in request

return self._request(node)

File "/home/devasc/.local/lib/python3.8/site-packages/ncclient/operations/rpc.py", line 348, in _request

raise self._reply.error

ncclient.operations.rpc.RPCError: {'type': 'application', 'tag': 'unknown-element', 'severity': 'error', 'info': '<?xml version="1.0" encoding="UTF-8"?><error-info xmlns="urn:ietf:params:xml:ns:netconf:base:1.0" xmlns:nc="urn:ietf:params:xml:ns:netconf:base:1.0"><bad-element>hostname</bad-element>\n</error-info>\n', 'path': '\n    /rpc/edit-config/config\n  ', 'message': None}

CÁCH TỰ ĐỘNG TÌM MODEL → TỰ BUILD XML ĐÚNG CHUẨN

STEP 1 — Luôn luôn xem CSR expose những YANG model nào

from ncclient import manager with manager.connect( host="192.168.74.152", port=830, username="cisco", password="cisco123!", hostkey_verify=False, look_for_keys=False, allow_agent=False ) as m: for cap in m.server_capabilities: print(cap)

# Configuration on CSR

From Question 1 to Question 5

Pexpect, Paramiko, Netmiko, Nornir, RESTCONF, NETCONF, Ansible

CSR1000v (192.168.74.152)

conf t !! -------------------------- !! 1. User + SSH + RSA Keys !! -------------------------- username cisco privilege 15 secret cisco123! ip domain-name lab.local crypto key generate rsa modulus 2048 line vty 0 4 login local transport input ssh !! -------------------------- !! 2. AAA (bắt buộc cho RESTCONF) !! -------------------------- aaa new-model aaa authentication login default local aaa authorization exec default local !! -------------------------- !! 3. Enable HTTP/HTTPS + RESTCONF !! -------------------------- ip http server ip http secure-server restconf !! -------------------------- !! 4. NETCONF !! -------------------------- netconf-yang !! -------------------------- !! 5. Kiểm tra port !! -------------------------- end show ip http server secure status show netconf-yang status

Check NETCONF enable: on CSR

show running-config | include netconf

Kết quả đúng phải thấy:

netconf-yang

✔ Check port 830 mở chưa:

show platform software yang-management process

Output :

confd : Running

nesd  : Running

ncsshd: Running

nginx : Running

→ NETCONF & RESTCONF OK.

On Devas check :

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

https://192.168.74.152/restconf/data/ietf-interfaces:interfaces

ON Devas

Show run | include netconf

Check Netconf lệnh 2 : 
curl -k -u cisco:cisco123! https://192.168.74.152/restconf/

Test 1 module nhỏ:

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

https://192.168.74.152/restconf/data/ietf-interfaces:interfaces

Các lệnh check

SSH OK? 	ssh cisco@192.168.74.152

RESTCONF OK?

curl -k -u cisco:cisco123! https://192.168.74.152/restconf/data/

2️⃣ Trên CSR1000v – cấu hình 1 lần là xong

✅ Những thứ BẮT BUỘC phải có (đúng như bạn ghi)

conf t

username cisco privilege 15 secret cisco123!

ip domain-name lab.local

crypto key generate rsa modulus 2048

line vty 0 4

login local

transport input ssh

✔ SSH cho Pexpect / Paramiko / Netmiko / Nornir

✅ AAA (cái này là chìa khóa của RESTCONF)

aaa new-model

aaa authentication login default local

aaa authorization exec default local

👉 Thiếu AAA = RESTCONF auto access-denied

👉 Bạn làm đúng rồi

✅ RESTCONF + HTTPS

ip http server

ip http secure-server

restconf

✅ NETCONF

netconf-yang

end

3️⃣ Check nhanh trên CSR (đúng – sai là biết liền)

show ip http server secure status

✔ Enabled

✔ Port 443

show running-config | include netconf

✔ Phải thấy netconf-yang

show platform software yang-management process

✔ Phải có:

confd : Running

nesd : Running

ncsshd: Running

nginx : Running

👉 Chỉ cần mấy dòng này = NETCONF & RESTCONF OK

4️⃣ Trên Devas – chỉ TEST, KHÔNG config gì thêm

🔹 SSH

ssh cisco@<CSR_IP>

✔ Vào được = pass Question 1–4 nền tảng

🔹 RESTCONF (test nhẹ, đừng query root nhiều)

✅ ĐÚNG – AN TOÀN

curl -k -u cisco:cisco123! \

-H "Accept: application/yang-data+json" \

https://<CSR_IP>/restconf/data/ietf-interfaces:interfaces

👉 Ra JSON = RESTCONF OK

# Lab 7

Nhớ tạo hosts ben6 ngoài thư mục playbooks

[ios] csr1 ansible_host=192.168.74.155 [ios:vars] ansible_connection=network_cli ansible_network_os=cisco.ios.ios ansible_user=cisco ansible_password=cisco123! ansible_become=yes ansible_become_method=enable ansible_become_password=cisco123!

playbooks/backup.yml

--- - name: Backup running-config hosts: ios gather_facts: no tasks: - name: Backup config (per-device file) cisco.ios.ios_config: backup: true backup_options: dir_path: backups

Tự tạo file dạng:

backups/CSR1kv_config_2025-xx-xx@xx-xx-xx

▶️ Run

ansible-playbook playbooks/backup.yml

2️⃣ Idempotent Base Config (playbooks/base_config.yml)

🎯 Mục tiêu

Cấu hình “nền”

Chạy nhiều lần không bị spam changed

Chuẩn bài kiểm tra: idempotence

playbooks/base_config.yml

--- - name: Base hardening (idempotent) hosts: ios gather_facts: no tasks: - name: Global settings cisco.ios.ios_config: lines: - service timestamps debug datetime msec - service timestamps log datetime msec - logging buffered 19999 - ip ssh version 2 - name: VTY lines (ensure SSH only) cisco.ios.ios_config: parents: line vty 0 4 lines: - exec-timeout 10 0 - transport input ssh - login local

ansible-playbook playbooks/base_config.yml

ansible-playbook playbooks/base_config.yml

🧩 Part 3 — Conditions & Facts (CHỐT)

🎯 Ý tưởng

ios_facts lấy trạng thái thật từ thiết bị

when làm “gate” → không đổi khi đã compliant

Chạy lại nhiều lần không phá config

1️⃣ Facts + Conditional config

playbooks/cond.yml

--- - name: Conditional hostname & gated tasks hosts: ios gather_facts: no tasks: - name: Gather network facts cisco.ios.ios_facts: - name: Append -LAB to hostname if not present cisco.ios.ios_config: lines: - "hostname {{ ansible_net_hostname }}-LAB" when: - ansible_net_hostname is defined - not ansible_net_hostname.endswith('-LAB') - name: Show final hostname cisco.ios.ios_command: commands: - show run ¦ include ^hostname register: h - name: Print hostname debug: var: h.stdout_lines # ===== Self-done task (a) ===== - name: Warn if IOS-XE version < 17 debug: msg: "⚠️ Warning: IOS-XE version is lower than 17.x" when: - ansible_net_version is defined - ansible_net_version is version('17.0', '<') # ===== Self-done task (b) ===== - name: Run task only if Gi1 is administratively up debug: msg: "GigabitEthernet1 is UP → safe to apply config" when: - ansible_net_interfaces is defined - ansible_net_interfaces['GigabitEthernet1'].enabled

✔️ Kỳ vọng khi chạy

Lần 1: changed

Lần 2: task hostname skipped → idempotent chuẩn bài

ansible-playbook playbooks/cond.yml