| EASTERN INTERNATIONAL UNIVERSITY SCHOOL OF COMPUTING AND INFORMATION TECHNOLOGY | Practice Assignment – Quarter …, 20…-20… Course Name: Special Topic 1 Course Code: CSE 484 Student’s Full Name: Hà Quang Minh Student ID: 1931220012 |
| --- | --- |

Lab 7

# Background

This 4-hour lab applies Ansible for network automation on Cisco IOS-XE (CSR1000v). You will: install Ansible and the network collections, prepare an inventory, run playbooks using ios_command/ios_config, use conditions/loops/templates, and follow better practices (backups, vault, lint). Topics mirror the chapter slides: overview/architecture, inventory, conditions, loops, templates, and recommended practices.

# Required Resources:

- 1 PC (Linux/macOS/WSL recommended), Python 3.10+, VS Code

- A reachable IOS-XE device (CSR1000v) with SSH/eAPI enabled

- Internet access to install: ansible, ansible-pylibssh, ansible-lint

- Ansible collections: cisco.ios, ansible.netcommon

- Credentials with privilege 15 (exec + enable)

Problem:

# Part 1 —Install & Verify

Goal: a working Ansible environment that can “talk” to your CSR.

### Create venv & install Ansible + libssh (agentless, Python-based):

python -m venv .venv && source .venv/bin/activate

pip install --upgrade pip

pip install ansible ansible-pylibssh ansible-lint

ansible --version

### Install required collections (vendor modules live in collections):

ansible-galaxy collection install cisco.ios ansible.netcommon

### Prepare working dir:

ansible-netlab/

├── ansible.cfg

├── inventories/

│   ├── hosts            # INI

│   └── host_vars/

│       └── csr1.yml     # per-device vars (YAML)

└── playbooks/

└── sanity.yml

### Create ansible.cfg (enable libssh & safe defaults)

[defaults]

inventory = inventories/hosts

host_key_checking = False

stdout_callback = yaml

gathering = explicit

timeout = 30

retry_files_enabled = False

[persistent_connection]

connect_timeout = 30

command_timeout = 60

### Inventory (inventories/hosts) using groups:

[ios]

csr1 ansible_host=192.168.2.50

### Host vars (inventories/host_vars/csr1.yml):

ansible_user: cisco

ansible_password: cisco

ansible_become: true

ansible_become_method: enable

ansible_become_password: cisco

# Connection & OS

ansible_network_os: cisco.ios.ios

ansible_connection: ansible.netcommon.network_cli

# Optional: use libssh transport (recommended in chapter)

# ansible_network_cli_ssh_type: libssh

### Sanity play (playbooks/sanity.yml) – gather facts and run a show:

- name: Sanity check IOS reachability

hosts: ios

gather_facts: no

tasks:

- name: Facts via ios_facts

cisco.ios.ios_facts:

- name: Show version

cisco.ios.ios_command:

commands: ["show version"]

register: showver

- name: Print model & version

debug:

msg: "Model={{ ansible_net_model }} Version={{ ansible_net_version }}"

### Run it:

ansible-playbook playbooks/sanity.yml

Evidence: screenshot of successful run (facts & show version echoed). (Ansible overview/architecture, agentless, inventory basics.)

### Self-Done Tasks

Break something intentionally (wrong password or wrong port), run the playbook, and capture the error message. Write a short explanation: What type of Ansible connection error was it?

My Solution

# Part 2 — Backups & Idempotent Config (0:30–1:20)

Goal: use ios_config to back up and make safe changes (idempotence).

### Backup play (playbooks/backup.yml) — automatic timestamped files:

- name: Backup running-config

hosts: ios

gather_facts: no

tasks:

- name: Backup config (per-device file)

cisco.ios.ios_config:

backup: true

backup_options:

dir_path: backups

Run: ansible-playbook playbooks/backup.yml

### Idempotent config (playbooks/base_config.yml) — no repeated changes:

Idempotent config (playbooks/base_config.yml) — no repeated changes:

- name: Base hardening (idempotent)

hosts: ios

gather_facts: no

tasks:

- name: Global settings

cisco.ios.ios_config:

lines:

- service timestamps debug datetime msec

- service timestamps log datetime msec

- logging buffered 19999

- ip ssh version 2

- name: VTY lines (ensure SSH only)

cisco.ios.ios_config:

lines:

- exec-timeout 10 0

- transport input ssh

- login local

parents: line vty 0 4

Run twice and confirm no changes on second run (idempotence).

### Self-Done Tasks

- Create a second backup play that only backs up configuration if the device hostname contains “LAB”:

  - Must use: when: "LAB" in ansible_net_hostname

Evidence: backup files under backups/, two run summaries (first “changed”, second “ok”).

# Part 3 — Conditions & Facts

Goal: use when with network facts to gate changes.

### Facts + conditional config (playbooks/cond.yml):

- name: Conditional hostname suffix

hosts: ios

gather_facts: no

tasks:

- name: Gather facts

cisco.ios.ios_facts:

- name: Append -LAB to hostname if not present

cisco.ios.ios_config:

lines:

- "hostname {{ ansible_net_hostname }}-LAB"

when: ansible_net_hostname is defined and

not ansible_net_hostname.endswith('-LAB')

- name: Show final hostname

cisco.ios.ios_command:

commands: ["show run | include ^hostname"]

register: h

- debug: var=h.stdout_lines

### Explain (2–3 sentences):

How when prevents repeated/unsafe changes?

Evidence: play output demonstrating skipped when already compliant.

### Self-Done Tasks

- Add a conditional task:

- If the device version is lower than 17.x, print a warning using a debug: task.

- Create a task that is executed only when:

- Interface Gi1 is administratively up (facts required).Use:

when: ansible_net_interfaces['GigabitEthernet1'].enabled

# Part 4 — Loops (standard & dictionary)

Goal: configure multiple items cleanly using loops.

### Standard loop to create VLANs (playbooks/vlans.yml):

- name: Create VLANs via loop

hosts: ios

gather_facts: no

vars:

vlans: [ {id: 10, name: APP}, {id: 20, name: DB}, {id: 30, name: WEB} ]

tasks:

- name: Ensure VLANs exist

cisco.ios.ios_config:

lines:

- "vlan {{ item.id }}"

- " name {{ item.name }}"

loop: "{{ vlans }}"

### Dictionary loop for interfaces (playbooks/intfs.yml)

- name: Access ports via dict loop

hosts: ios

gather_facts: no

vars:

access_ports:

- { name: GigabitEthernet1, vlan: 10, desc: "APP Uplink" }

- { name: GigabitEthernet2, vlan: 20, desc: "DB Uplink" }

tasks:

- name: Configure access interface

cisco.ios.ios_config:

parents: "interface {{ item.name }}"

lines:

- "description {{ item.desc }}"

- "switchport mode access"

- "switchport access vlan {{ item.vlan }}"

- "no shutdown"

loop: "{{ access_ports }}"

Evidence: play recaps showing looped changes; show vlan brief and show run interface outputs.

Submission: Must include:

- a pdf report file containing your information (student id, name), and images of the diagrams, codes, answers, evident,…

- and all source code files (if any)

in a zipped (.zip or *.rar) file to Moodle